CONCEPT DESIGN · VOLUME 01

JEN TUAN CHAU · HERO VILLA CONCEPT

Single Manor Residence — Caye Sereno, Khu Biệt Thự Lâm Ngọc, Tuần Châu, Hạ Long

A colonial-tropical peer to Capella Sentosa Manors,

set into the existing Caye Sereno SEA Type 2 plan with L1 extended to match the ground-floor footprint.

Prepared for  ·  Công ty TNHH Jen Tuan Chau

Base drawings  ·  Transform Architecture, Caye Sereno SEA Type 2 (06/2015)

Reference  ·  Capella Singapore — Sentosa Manors (Foster + Partners / Matthew Shang)

Constraint source  ·  Báo cáo thẩm định Jen Tuan Chau (2025)

April 2026 · Issue for Client Review

# 01 · Executive Summary

This document sets out the concept design for a single hero manor villa on the Jen Tuan Chau estate (Khu Biệt Thự Lâm Ngọc, Tuần Châu island, Hạ Long Bay). The design is positioned as a peer to the Capella Sentosa Manors in Singapore — a low-rise, bay-facing, colonial-tropical residence that sits inside a UNESCO World Heritage buffer zone without competing with it.

The brief is threefold: (1) retain the plan and setting-out of the existing Caye Sereno SEA Type 2 villa prepared by Transform Architecture in June 2015 so that approvals, setbacks and the 1/500 master plan carry through unchanged; (2) extend the first floor (L1) to match the full ground-floor footprint so that usable GFA is maximised under the approved envelope; (3) replace the contemporary modernist language of the as-built drawings with a colonial-tropical language — pitched terracotta roof, rendered walls, deep verandas, dark timber shutters — that is directly readable as a Capella peer.

### Headline numbers

| Metric | Existing (Caye Sereno SEA Type 2) | Proposed (Hero Villa) | Δ |
| --- | --- | --- | --- |
| Plot area | ~840 m² (28.05 × 30.00 m) | ~840 m² (unchanged) | — |
| GF enclosed area | ~300 m² | ~300 m² | 0 |
| L1 enclosed area | ~152 m² (partial) | ~300 m² (full match) | +148 m² |
| Total GFA | ~452 m² | ~600 m² | +148 m² (+33%) |
| Ridge height | ~7.00 m (flat roof +6.97) | ≤9.00 m (pitched, compliant) | within envelope |
| Bedrooms | 4 suites | 4 suites + study + staff | program upgrade |
| Style | Contemporary modernist | Colonial-tropical (Capella peer) | re-clad |

Against the Capella Sentosa Manor benchmark of 434–436 m², the proposed villa is ~38% larger. This is the direct effect of the user-confirmed decision “Full match — L1 = GF footprint”, and satisfies the brief to build to the maximum area permitted by the approved 1/500 envelope.

# 02 · Site, Approvals and Constraints

The site sits on the eastern tip of Tuần Châu island, on plots covered by Land Use Right Certificates Bi 462036 (7,774 m²) and Bi 462037 (8,054 m²) — a combined 15,828 m² forming the Lâm Ngọc villa zone. The approved sub-division (Quyết định 1112/2012, master plan 1/500) contemplates eighteen ultra-premium villa plots arranged along the southern (bay-facing) edge, with service access from the north.

## 2.1 Statutory envelope

| Approval / Constraint | Reference | Effect on design |
| --- | --- | --- |
| 1/500 Master Plan — Lâm Ngọc | QĐ 1112 · 11-05-2012 | Fixes plot, setbacks, 2–3 floor ceiling |
| 1/2000 Master Plan — Cảng Ngọc Châu | QĐ 5222 · 12-12-2019 | Context / road alignment |
| Investment Certificate (5th revision) | 10-07-2025 | Project remains live; no expiry concern |
| Business classification | QCVN 06:2022/BXD — F1.2 | ≤3 floors residential/tourist lodging |
| UNESCO Ha Long Bay buffer | 306 km² buffer zone | Low rise, muted colour, no reflective finishes |
| Environmental commitment | §14 — DTM/CKBVMT accepted | No further EIA trigger at villa scale |
| Building permit | §13 — exempt (approved 1/500) | No GPXD required; construction notice only |

## 2.2 Risk register carried forward from the appraisal

The Jen Tuan Chau due-diligence appraisal (báo cáo thẩm định, 2025) scores the project MEDIUM-HIGH RISK on a 1R+3Y+2G traffic-light basis. The design must not worsen any of those flags:

- UNESCO buffer zone (🔴 red) — retain low ridge (≤9 m), muted palette, terracotta roof matched to Capella's copper heritage so the villa reads as landscape, not skyline.

- Land-use fees & CCC (🟡 yellow) — design keeps GFA inside the approved 1/500 envelope; no rezoning needed.

- JVA governance (🟡 yellow) — deliverable is style-neutral to the Jen Cap × Đào Ngọc split; can be built by either party.

- Construction history (🟡 yellow) — PHG completion dated 19-06-2015; new works treated as refurbishment + L1 extension, not fresh ground-up.

- LURC clarity (🟢 green) — Bi 462036 + Bi 462037 preserved.

- Financial records (🟢 green) — six years of audited FS support the capex case.

# 03 · Precedent — Capella Sentosa Manors

Capella Singapore occupies two restored 1880s colonial “black-and-white bungalows” on Sentosa, wrapped by a new low-rise hotel wing designed by Foster + Partners (restoration completed 2009; interiors by Matthew Shang Design Office for the 2023 refresh). The three Sentosa Manors are three-bedroom, 434–436 m² private residences set inside the hotel's landscape, each with its own pool, garden and colonial-plantation architecture — deep overhanging pitched roofs, white-rendered walls, dark teak shutters, wraparound verandas, copper-to-terracotta roof tones, and an ivory/charcoal/brass interior palette punctuated by tropical planting.

## 3.1 Design language transferred to Jen Tuan Chau

| Capella Sentosa Manor attribute | Application at Jen Tuan Chau |
| --- | --- |
| Low single/two-storey silhouette | Retain ~7–9 m ridge; two storeys with pitched roof over flat-roof lines |
| White-rendered walls, dark timber shutters | Replace STW2 stone cladding on upper floors with lime-washed render; SDET-075 bamboo screens become teak louvered shutters |
| Copper / terracotta pitched roof | Introduce pitched clay-tile roof over the existing ArAD flat-roof datum; copper gutter + ridge |
| Deep verandas wrapping bay elevations | 2.4 m cantilever veranda along south and west elevations |
| Colonial symmetry around central foyer | Keep existing gridline C as central axis; mirror plan around it |
| Neutral ivory/charcoal/brass interior | Replace contemporary palette of the SEA Type 2 with lime-washed walls, teak joinery, aged brass, raffia textiles |
| Tropical botanical planting (frangipani, heliconia) | Foreground planting on pool terrace; mature frangipani flanking entry court |
| Private pool, outdoor sala, BBQ | Retain existing pool; add a sala pavilion on the east lawn |

The goal is not pastiche. Capella's authority comes from the honesty of the bungalow plan — a thick central spine with verandas on both long sides, cross-ventilated rooms, tall ceilings and a roof that does the aesthetic work. The existing Caye Sereno plan already has the right bones (a linear main block with secondary service wing); re-cladding the skin and pitching the roof is enough to carry the reference convincingly.

# 04 · Base Drawings — Caye Sereno SEA Type 2

The as-built package by Transform Architecture (sheet set ArAD, June 2015, executed by Phú Hưng Gia) provides the setting-out, structural grid and finishes library that the concept re-uses. Dimensions are preserved exactly; only the envelope, roof and L1 extent are modified.

## 4.1 Dimensional inventory

| Parameter | Value | Source sheet |
| --- | --- | --- |
| Long-axis grid | 3,850 + 5,500 + 5,500 + 7,150 + 6,050 = 28,050 mm | PLN-012 |
| Plot depth | 11,550 mm (living bay); 14,350 mm (dining wing) | PLN-012 / PLN-013 |
| GF FFL | +2.930 m (SL +2.770) | PLN-012 |
| L1 FFL | +3.180 m | PLN-013 |
| Existing roof FFL | +6.970 m (SL +6.800) | PLN-014 |
| Overall height (existing) | ~7.00 m (flat roof) | ELE-031 / ELE-033 |
| Floor-to-floor GF→L1 | ~3.55 m | SEC-041 |

## 4.2 Sheet carry-over

The ArAD package inventoried below is treated as the reference baseline. Sheets marked ● are updated under this concept; sheets marked ○ are retained verbatim for coordination with civil, MEP and landscape packages.

| Sheet group | Sheets | Carry-over |
| --- | --- | --- |
| Plans | PLN-012 GF · PLN-013 L1 · PLN-014 Roof | ● Updated (L1 extended, roof pitched) |
| Floor finishes | FFL-015 GF · FFL-016 L1 | ● Updated (interior palette re-set) |
| Ceiling / RCP | CEL-017 | ● Updated (coffered / tongue-groove) |
| Elevations | ELE-031/032/033 · 1A–4B (eight elevations) | ● Updated (colonial re-clad) |
| Sections | SEC-041/042/043 · A–G (seven sections) | ● Updated (pitched roof profile) |
| Details | SDET-075 bamboo screen · AD-series glazing · STW2 stone base | ○ Retained; re-issued in teak / clay tile / stone |

# 05 · Design Strategy

## 5.1 Five moves

Move 1 · Extend L1 to the full GF footprint.  The existing plan leaves a ~148 m² open roof terrace over the dining and primary-suite wings. This is the single biggest GFA opportunity; the concept converts that void into conditioned floor, giving three additional guest suites plus an upper lounge while keeping ridge height below 9 m.

Move 2 · Pitch the roof.  The flat ArAD roof at +6.97 is replaced with a 25° hipped clay-tile roof rising to ridge ~+9.00. The pitched profile reads as Capella-peer at 100 m distance and is the dominant image in bay-view photography.

Move 3 · Re-clad in colonial white.  Stone base (retained where existing) to +3.18; white lime-washed render to underside of eaves; teak louvered shutters on all openings to the south and west; deep reveals (300 mm) to give plasticity in raking sunlight.

Move 4 · Wrap the bay elevation in veranda.  A 2.4 m cantilevered veranda along the south (bay-facing) and west elevations on both GF and L1 — the classic colonial bungalow move — shades the glazing, protects the render from rain and gives every principal room a direct outdoor-to-bay transition.

Move 5 · Re-interior around an ivory/teak/brass palette.  Replace the contemporary interior with rough-plastered walls in warm ivory, herringbone teak floors, woven raffia ceiling panels, aged-brass hardware, linen upholstery in oat and ink, art and textiles sourced from Hạ Long / Đông Hồ craft traditions.

## 5.2 Program brief

| Level | Rooms | Area (approx.) |
| --- | --- | --- |
| GF (~300 m²) | Porte-cochère · formal entry foyer · formal living (bay) · dining (bay) · family living · study / library · open kitchen · service kitchen · pantry · primary suite + dressing + ensuite · powder · maid · utility · south + west veranda (2.4 m) | 300 m² enclosed + ~120 m² covered veranda |
| L1 (~300 m², new full match) | Guest suite 1 (bay, king) · Guest suite 2 (bay, king) · Guest suite 3 (garden, twin) · guest lounge / gallery · reading nook · upper veranda (bay) | 300 m² enclosed + ~90 m² covered upper veranda |
| Roof | Hipped clay-tile, 25°, copper ridge + gutters; ridge vent; concealed PV option at rear slope | — |
| Site | Infinity pool (retained, re-edged in tumbled basalt) · pool deck in teak · detached sala pavilion · outdoor kitchen / BBQ · service yard at north · motor court with frangipani pair · gardener's store | ~540 m² external program |

# 06 · Massing, Plans, Elevations

## 6.1 Massing diagram (narrative)

Looking south from the motor court, the villa presents a long linear mass parallel to the bay. The GF base is a continuous white-rendered volume of 28.05 × 11.55 m with a projecting dining wing to the east (+2.8 m). The L1, now extended to the full GF outline, sits directly above with a 300 mm overhang forming the veranda ceiling. A 25° hipped clay-tile roof covers the whole, peaking centrally on gridline C and hipping to the east and west gables. The roof line reads as a single unified profile — the single most important change from the existing flat-roofed scheme.

## 6.2 Ground floor plan — narrative

Arrival is from the north onto a paved motor court flanked by a pair of mature frangipani trees. The porte-cochère sits on gridline C–D; beyond it, a double-height foyer with coffered teak ceiling opens left to the formal living room (bay view, full-height sliding glass to south veranda) and right to the formal dining (bay view, 10-seat teak table). A service spine at the north edge contains the pantry, service kitchen, maid's room and utility — keeping back-of-house invisible from the bay elevation. The primary suite occupies the west end of the GF, with its own covered terrace stepping directly down to the pool. The east end carries the study/library (half-panelled teak, writing desk, three armchairs) and a guest powder room.

## 6.3 First floor plan — the extension

L1 mirrors the GF outline (28.05 × 11.55 m plus the 14.35 m dining-wing projection). The plan is organised around a central axial corridor on gridline C running east–west. Two principal guest suites (Suite A and Suite B) sit on the south, bay-facing wall, each with a private slice of the 2.4 m upper veranda. Suite C (twin-bed, garden view) sits on the north. The central ‘gallery' widens over the GF foyer into a double-height reading atrium with a roof lantern (the only break in the hipped roof). At the east end, what was the open dining-roof terrace becomes the guest lounge and small library.

## 6.4 Elevations

| Elevation | Role | Key notes |
| --- | --- | --- |
| South (1A/1B) — bay | Hero image | Full 28 m veranda run; 5 bays of teak louvered shutters; L1 over-sailing the GF at 2.4 m; clay-tile roof pitching down to +6.50 eave |
| East (2A/2B) — dining wing | Secondary hero | Projecting gable end with a jack-arch teak window to the dining room; sala pavilion in foreground |
| North (3A/3B) — arrival | Arrival | Porte-cochère with stone base, double-leaf teak doors, clerestory lantern above foyer; service yard screened by a low masonry wall |
| West (4A/4B) — primary suite | Private | Primary suite terrace; roof hips in toward centre; solid render with two shuttered windows only |

## 6.5 Section A–A (transverse)

Cut north–south through gridline D, the section reads (from bay to service): 2.4 m veranda · 5.5 m formal living (GF) / guest suite (L1) · 1.5 m circulation spine · 2.2 m service zone. Floor-to-floor GF = 3.55 m; L1 = 3.20 m; roof ridge at central gridline = +9.00 m. The pitched roof geometry is set so that every principal bay-facing room has a minimum 3.0 m ceiling height at the head of the bed; the ridge line is concealed from the pool deck by the upper veranda eave.

# 07 · Materials, Finishes, Colour

## 7.1 External palette

| Element | Material | Colour / finish | UNESCO-buffer note |
| --- | --- | --- | --- |
| Roof | Hand-made clay pan tile, hipped, 25° | Weathered terracotta (Capella copper match) | Muted, matte, no metallic glint |
| Ridge + gutters | Patinated copper | Verdigris over 5 years | Aged finish at handover — pre-weathered |
| Walls (upper) | Lime-wash render over masonry | Warm ivory (NCS S 1005-Y20R) | Non-reflective, no colour competing with karst |
| Walls (base to +3.18) | Retained STW2 split-face basalt | Natural dark charcoal | Existing finish, no change |
| Shutters / louvres | FSC teak, oiled | Ink-charcoal stain, matte | Directly replaces SDET-075 bamboo |
| Veranda ceiling | Tongue-and-groove teak | Natural oil | Softens overhang in photographs |
| Glazing | Retained AD-series, bronze frames | Neutral solar tint 20% | No reflective coatings |
| Veranda floor | Reclaimed teak deck | Natural silver-grey weathering | Timber reads as landscape |
| Pool edge | Tumbled Indonesian basalt | Charcoal, honed | Matches base wall |
| Paths & motor court | Sawn basalt setts with grass joints | Charcoal + green | Permeable paving |

## 7.2 Interior palette

| Zone | Walls | Floor | Ceiling | Feature |
| --- | --- | --- | --- | --- |
| Entry foyer | Venetian-plaster ivory | Tumbled travertine | Coffered teak | Aged-brass lantern, 1.6 m dia |
| Formal living | Rough-plaster ivory, dado rail | Herringbone teak | Raffia-panel between teak joists | Linen sofas, oat; ink velvet armchairs |
| Formal dining | Ivory with teak wainscoting | Herringbone teak | Coffered, raffia infill | 10-seat teak table, rattan chairs |
| Kitchen (open) | Ivory with marble splashback | Tumbled travertine | Flush plaster | Honed Carrara island, brass hardware |
| Primary suite | Warm ivory, textile headboard wall | White oak | Flush plaster | Teak four-poster, raffia pendant |
| Primary bath | Honed Carrara panels | Honed Carrara | Flush plaster | Freestanding brass tub, bay view |
| Guest suites (L1) | Rough-plaster ivory | White oak | Flush plaster with exposed tie-beam | Shutter-framed bay view from bed |
| Guest lounge (L1) | Half-panel teak, ivory above | Herringbone teak | Coffered teak | Reading lamp, library wall |

# 08 · Interior & Landscape Narrative

## 8.1 Arrival sequence

Guests enter from the north through a 4 m wide motor court of sawn basalt with grass joints, flanked by two mature frangipani in raised stone planters. The porte-cochère is a single stone-clad volume with a pair of solid teak doors (2.4 m high, recessed teak panelling, aged-brass pulls). Crossing the threshold, the foyer rises to a double-height space capped by a coffered teak ceiling and a 1.6 m aged-brass lantern; a central axial vista draws the eye through the living and dining rooms to the bay beyond. The arrival sequence is staged for photography — 12 m of compressed-to-released ceiling height, charcoal-to-ivory contrast, and a first sight of the bay framed by teak mullions.

## 8.2 The bay room (formal living)

The formal living room is oriented along the south elevation — 11 m wide, 6 m deep, 3.4 m ceiling. Four pairs of full-height sliding teak-and-glass doors open onto the veranda. The central seating group is two 3.2 m linen sofas in oat facing each other across a reclaimed-teak coffee table, bracketed by two ink-velvet armchairs. A paired fireplace wall of honed travertine sits opposite the bay (non-functional in climate terms, but giving the room an axial anchor). Raffia-panel ceiling between exposed teak joists; vintage Turkish rug in faded rust; two aged-brass pendant lanterns. At dusk, the shutters half-closed throw horizontal-striped shadows across the floor.

## 8.3 Primary suite

The primary suite occupies the west end of the GF, with its own covered terrace stepping directly onto the pool deck. A teak four-poster bed faces south to the bay; the textile headboard wall is covered in handwoven raffia. A pair of pivoting teak shutters frames the bay view from bed. The bathroom is a single honed-Carrara volume: a freestanding aged-brass tub sits on the south wall under a 1.2 × 1.2 m pivot window, giving a bath-with-bay-view moment that reads in Instagram format. The dressing room is panelled in teak with aged-brass inlays.

## 8.4 Landscape — the bay terrace

The existing pool is retained but re-edged in tumbled basalt and extended by 1.5 m south toward a freshly landscaped planting bed of heliconia, bird-of-paradise and tropical grasses. A detached sala pavilion (3.6 × 3.6 m, pitched clay-tile roof to match the main house, open on four sides, day bed + brass drinks cart) sits on the east lawn as a shaded outdoor living room. An outdoor kitchen with a wood-fired grill is tucked into the south-east corner of the house. Mature mango and sea-almond trees edge the west boundary, giving afternoon shade to the primary suite terrace and framing the bay view in long-tail.

# 09 · Compliance Checklist

| # | Requirement | Source | Status |
| --- | --- | --- | --- |
| 1 | Number of floors ≤ 3 (F1.2) | QCVN 06:2022/BXD | ✓ GF + L1 only |
| 2 | GFA within approved 1/500 envelope | QĐ 1112/2012 | ✓ L1 matches GF outline; no plot extension |
| 3 | Setbacks per 1/500 | QĐ 1112/2012 | ✓ Existing Caye Sereno setbacks retained unchanged |
| 4 | Ridge ≤ 9.0 m | UNESCO buffer convention | ✓ Central ridge +9.00 m; eave +6.50 m |
| 5 | Muted, non-reflective exterior palette | UNESCO buffer convention | ✓ Terracotta + ivory + basalt + teak |
| 6 | No new land-use fee trigger | §5 LU fee receipts on file | ✓ No plot change, no re-zoning |
| 7 | Environmental commitment still satisfied | §14 CKBVMT accepted | ✓ Refurb + L1 extension within approved scope |
| 8 | Building permit exemption preserved | §13 GPXD-miễn | ✓ 1/500 approval still governs |
| 9 | LURC parcels unchanged | Bi 462036 / Bi 462037 | ✓ No sub-division |
| 10 | JVA obligations neutral | JVA 06-07-2012 + addenda | ✓ Design deliverable is style-neutral |

## 9.1 Open items for engineering coordination

- Structural — confirm existing columns/beams can take the new L1 slab over the former roof terrace (likely; the existing structure was sized for a full L1 floor in the original Transform scheme).

- Structural — new pitched roof truss span ≤ 11.55 m (primary) / 14.35 m (dining wing); teak glulam feasible, steel backup acceptable.

- MEP — cooling load grows ~33% with new L1 conditioned area; confirm existing chiller plant capacity or size an additional compressor.

- Drainage — pitched roof increases rainwater volume; new copper gutters + downpipes + sub-surface soakaway in the side setbacks.

- Fire — two escape staircases required at ≤25 m travel distance; existing single stair to be supplemented with a secondary stair at the dining-wing east gable.

- Acoustics — raffia ceiling panels specified as perforated backing board to maintain Rw ≥ 45 dB between guest suites.

# 10 · Deliverables Register

This concept design is one of three co-produced deliverables. The full package is listed below for the Jen Tuan Chau team:

| # | Deliverable | Format | Audience |
| --- | --- | --- | --- |
| 1 | Concept Design — this document | .docx | Board, JVA partners, planning reviewer |
| 2 | Visual package — interactive site/plan/elevation viewer | .html | Board, marketing, investor deck |
| 3 | External-render prompt pack | .md | Visualisation studio / Midjourney / Flux |

## 10.1 Prompt pack — summary

- Hero exterior (south-west corner, golden hour, veranda lit from within, bay in background).

- Arrival / porte-cochère at dusk (copper lanterns, frangipani, teak doors).

- Formal living room — bay view through open shutters, linen sofa, raffia ceiling.

- Primary suite — four-poster bed, shutter-framed bay view, warm morning light.

- Pool terrace — infinity edge to bay, sala pavilion mid-frame, planting foreground.

## 10.2 Next steps

- Client review of this concept package — 5 working days.

- Structural feasibility sign-off on L1 slab over former roof terrace — 10 working days.

- Schematic Design (SD) package authorisation — target 1 June 2026.

- Design Development (DD) including MEP, FF&E, landscape — target 1 September 2026.

- Tender and construction authority notice (no GPXD required per §13) — target Q4 2026.

# Appendix A · Capella Sentosa Manor Spec Transfer

| Capella Sentosa Manor | Value | Jen Tuan Chau equivalent |
| --- | --- | --- |
| Typology | 3-bed colonial manor | 4-bed + study colonial manor |
| GFA (interior) | 434–436 m² | ~600 m² (+38%, L1 full match) |
| Bedrooms | 3 en-suite | 4 en-suite (1 primary + 3 guest) |
| Living areas | 2 (formal + family) | 2 (formal + family) + study |
| Pool | Private pool | Existing Caye Sereno pool, re-edged |
| Pavilion | Outdoor sala | Detached sala pavilion, east lawn |
| Roof | Pitched clay tile, copper ridge | Pitched clay tile, copper ridge (25°) |
| Walls | White lime-wash render | White lime-wash render (NCS S 1005-Y20R) |
| Shutters | Dark teak louvered | FSC teak, ink-charcoal stain |
| Interior signature | Ivory + teak + brass + raffia | Ivory + teak + brass + raffia |
| Landscape | Frangipani, heliconia, palms, mature trees | Frangipani pair at entry; heliconia + bird-of-paradise at pool; mango + sea-almond at west boundary |

# Appendix B · Area & Dimension Cross-Check

All dimensions below are taken directly from ArAD.pdf (Transform Architecture, 06/2015). Areas are carried forward to the Hero Villa scheme.

| Zone | Dimensions (mm) | Area (m²) |
| --- | --- | --- |
| GF main block (living/dining) | 13,200 × 11,550 | 152.46 |
| GF dining wing projection | 14,350 × 9,600 + service fillet | ≈ 147.80 |
| GF enclosed — TOTAL | — | ≈ 300 |
| L1 (existing, Suites 3 + 4 + baths) | 13,200 × 11,550 (partial) | ≈ 152 |
| L1 extension new (over former terrace) | 14,350 × 9,600 + fillet | ≈ 148 |
| L1 enclosed — TOTAL (proposed) | Full match to GF | ≈ 300 |
| Grand Total GFA | — | ≈ 600 |
| Covered veranda GF (south + west) | 2.4 m × ~50 m | ≈ 120 |
| Covered veranda L1 (south) | 2.4 m × ~38 m | ≈ 90 |
| Pool surface (retained, re-edged) | existing + 1.5 m extension | ≈ 54 |
| Pool deck + sala pavilion lawn | — | ≈ 220 |

The area calculation confirms the headline figure of ~600 m² total GFA, ≈38% larger than the Capella Sentosa Manor benchmark. Because the L1 is extended within the existing GF outline, the plot and 1/500 envelope are unchanged — no re-approval is required.